3GPP TSG-SA WG3 LI Meeting # 8	Tdoc S3LI01_127

Aspen, Colorado, USA. 30 October – 1 November 2001

| CR-Form-v4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 33.108 |  |  |  | CR | CRNum |  |  |  rev |  |  |  | - |  |  Current version: |  |  |  |  | 0.1.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  |  |  | Radio Access Network |  |  |  |  |  |  |  |  | Core Network |  |  |
| Title:  |  | Cleanup for the definitions and mappings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Nokia corporation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 22 October 2001 |  |  |  |  |  |
| Category:  |  | D |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | Rel-5 |  |  |  |  |  |
|  |  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | TS 33.108 inherited the following mismatch between stage 2 LI specific identifiers and respective stage 3 parameters from ETSI ES 201 671: - CID identifier definition and its mapping to ASN.1 parameter is incorrect; - CIN identifier’s mapping to ASN.1 parameter is incorrect; - Correlation Number identifier definition is missing. Mapping of the Correlation Number parameter to the Correlation Number identifier is incorrect; - Both SGSN and GGSN may provide for the interception. Therefore, the name of the intercepting node parameter, the ‘sgsnAddress’ in not quite accurate. It is proposed to align stage 2 and stage 3 definitions. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | The Correlation Number identifier is defined in a new section 6.1.3. CID identifier is mapped to the ‘communicationIdentifier’ parameter. CIN identifier is mapped to the 'communication-Identity-Number' parameter. The name of the ASN.1 parameter, the ‘sgsnAddress’ is replaced by a generic one: ‘interceptingNodeAddress’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | Misalignment would lead to incorrect implementation. That could result in different encoding of HI2 and HI3 parameters. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 6.1; 6.5; Annex C.2; Annex B.4. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  | ‘S3LI01_126 DP on HI2 harmonization with HI3’ describes the reasons for change in more detail. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at: http://www.3gpp.org/3G_Specs/CRs.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://ftp.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2001-03 contains the specifications resulting from the March 2001 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

## 6.1	Identifiers

Specific identifiers are necessary to identify a target for interception uniquely and to correlate between the data, which is conveyed over the different handover interfaces (HI2 and HI3). The identifiers are defined in the subsections below.

For the delivery of CC and IRI the SGSN or GGSN provide correlation numbers and target identities to the HI2 and HI3. The correlation number is unique per PDP context and is used to correlate CC with IRI and the different IRI’s of one PDP context

### 6.1.1	Lawful interception identifier (LIID)

For each target identity related to an interception measure, the authorized NWO/AP/SvP operator shall assign a special Lawful Interception Identifier (LIID), which has been agreed between the LEA and the NWO/AP/SvP.

Using an indirect identification, pointing to a target identity makes it easier to keep the knowledge about a specific interception target limited within the authorized NWO/AP/SvP operators and the handling agents at the LEA.

The LIID is a component of the CC delivery procedure and of the IRI records. It shall be used within any information exchanged at the handover interfaces HI2 and HI3 for identification and correlation purposes.

The LIID format shall consist of alphanumeric characters. It might for example, among other information, contain a lawful authorization reference number, and the date, when the lawful authorization was issued.

The authorized NWO/AP/SvP shall either enter a unique LIID for each target identity of the interception subject or a single LIID for multiple target identities all pertaining to the same interception subject.

If more than one LEA intercepts the same target identity, there shall be unique LIIDs assigned relating to each LEA.

### 6.1.2	Communication identifier (CID)

For each communication or other activity relating to a target identity a CID is generated by the relevant network element. The CID consists of the following two identifiers:

-	 Network identifier (NID);

-	Communication Identity Number (CIN).

.

NOTE:	If interception has been activated for both parties of the packet data communication both CC and IRI will be delivered for each party as separate intercept activity.

#### 6.1.2.1	Network identifier (NID)

The network identifier is a mandatory parameter; it should be internationally unique. It consists in one or both of  the following two identifiers. It is mandatory that one of them is used.

1)	NWO/AP/SvP- identifier (optional):
Unique identification of network operator, access provider or service provider.

2)	Network element identifier NEID (optional):
The purpose of the network element identifier is to uniquely identify the relevant network element carrying out the LI operations, such as LI activation, IRI record sending, etc.

A network element identifier may be an IP address or other identifier.

#### 6.1.2.2	Communication identity number (CIN)

The communication identity number is a temporary identifier of an intercepted communication relating to a specific target identity.

### Correlation Number (CN)

The Correlation Number (CN) identifier is used for the following purposes :

- CN uniquely identifies the CC data associated with the PDP context

- CN is used to correlate CC with IRI

- CN is used to correlate different IRI's of one PDP context

DF3 shall use the same Correlation Number parameter, which DF2 has used for IRI delivery.

*** Next Modification ***

Table 2: Mapping between Events information and IRI information

| parameter | description | HI2 ASN.1 parameter |
| --- | --- | --- |
| observed MSISDN | Target Identifier with the MSISDN of the target subscriber (monitored subscriber). | partyInformation or msISDN |
| observed IMSI | Target Identifier with the IMSI of the target subscriber (monitored subscriber). | partyInformation or imsi |
| observed IMEI | Target Identifier with the IMEI of the target subscriber (monitored subscriber) | partyInformation or mei |
| observed PDP address | PDP address used by the target.. | partyInformation or pDP-address-allocated-to-the-target |
| event type | Description which type of event is delivered: PDP Context Activation, PDP Context Deactivation,GPRS Attach, etc. | gPRSevent |
| event date | Date of the event generation in the GSN | timeStamp |
| event time | Time of the event generation in the GSN |  |
| Access Point Name (APN) | The APN of the access point | partyInformation or aPN |
| PDP type | This field describes the PDP type as defined in TS GSM 09.60, TS GSM 04.08, TS GSM 09.02 | partyInformation or pDP-type |
| communication direction [Editor note: FFS] | This field indicates whether the PDP context activation or deactivation is MS directed or network initiated. | intercepted-Call-Direct |
| Communication Identifier (CID) | This field is used to uniquely identify an intercepted communication. | communicationIdentifier |
| Communication Identity Number (CIN) | This field can be used as a temporary identifier of an intercepted communication relating to a specific target identity | communication-Identity-Number' |
| Correlation Number (CN) | Unique number for each PDP context delivered to the LEMF, to help the LEA, to have a correlation between each PDP Context and the IRI. | gPRSCorrelationNumber |
| Lawful Interception Identifier (LIID) | Unique number for each lawful authorization. | LawfulInterceptionIdentifier |
| CGI (Cell Global ID) | Cell number of the target; for the location information | locationOfTheTarget or globalCellId |
| Routing Area Code | Routing-area-code of the target defines the Routing Area in a GPRS-PLMN | locationOfTheTarget or rAId |
| SMS | The SMS content with header which is sent with the SMS-service | sMS |
| failed context activation reason | This field gives information about the reason(s) for failed context(s) activation of the target subscriber. | gPRSOperationErrorCode |
| failed attach reason | This field gives information about the reason(s) for failed attach attempts of the target subscriber. | gPRSOperationErrorCode |
| SGSN Address | This field provides the address (IP or X.25) of the SGSN containing the IAP. | sgsnAddress |
| Server center address | This field identifies the address of the relevant server within the calling (if server is originating) or called (if server is terminating) party address parameters for SMS-MO or SMS-MT. | serverCenterAddress |

NOTE:	LIID parameter must be present in each record sent to the LEMF.

*** Next Modification ***

Table C.2: Information elements in the CC header

| Mode | Type | Length | Value |
| --- | --- | --- | --- |
| M | 130 | 2 | Version = the version number of the format version to be used. This field has a decimal value, this enables version changes to the format version. The values are allocated according to national conventions. |
| O | 131 | 2 | HeaderLength = Length of the CC-header up to the start of the payload. octets. (This field is optional since it is useful only in such cases that these information elements would be transferred without a dynamic length encapsulation that contains all the length information anyway. This field could be needed in case of e.g. adapting to a local encapsulation convention.) |
| O | 132 | 2 | PayloadLength = Length of the payload following the CC-header. (This field is optional since it is useful only in such cases that these information elements would be transferred without a dynamic length encapsulation that contains all the length information anyway. This field could be needed in case of e.g. adapting to a local encapsulation convention.) |
| M | 133 | 1 | PayloadType = Type of the payload, indicating the type of the CC. Type of the payload. This field has a decimal value. The possible PDP Type values can be found in the standards (e.g.3GPP TS 29.060 [17]). The value 255 is reserved for future PDP Types and means: "Other". |
| O | 134 | 4 | PayloadTimeStamp = Payload timestamp according to intercepting node. (Precision: 1 second, timezone: UTC). Format: Seconds since 1970-01-01 as in e.g. Unix (length: 4 octets). |
| C | 137 | 1 | PayloadDirection = Direction of the payload data. This field has a decimal value 0 if the payload data is going towards the target (ie. downstream), or 1 if the payload data is being sent from the target (ie. upstream). If this information is transferred otherwise, e.g. in the protocol header, this field is not required as mandatory. If the direction information is not available otherwise, it is mandatory to include it here in the CC header. |
| O | 141 | 4 | CCSeqNumber = Identifies the sequence number of each CC packet during interception of the target. This field has a 32-bit value. |
| M | 144 | 8 or 20 | CorrelationNumber. Identifies an intercepted session of the observed target. This can be implemented by using e.g. the Charging Id (4 octets, see [14]) with the (4-octet/16-octet) Ipv4/Ipv6 address of the PDP context maintaining GGSN node, attached after the first 4 octets. |
|  |  |  | <Possible future parameters are to be allocated between 145 and 253.> |
| O | 254 | 1-25 | LIID = Field indicating the LIID as defined in this document. This field has a character string value, e.g. "ABCD123456". |
| O | 255 | 1-N | PrivateExtension = An optional field. The optional Private Extension contains vendor or LEA or operator specific information. It is described in the document 3GPP TS 29.060 [17]. |

*** Next Modification ***

...

interceptingNodeAddress 			[21] DataNodeAddress OPTIONAL,

...